UWB DSTO Data Sheet

version: 1.0

by: Lin

目录

UWB DSTO Data Sheet	1

1. System Introduction	1

1.1 System Introduction	1

1.2 System Features	2

1.3 Application objects	2

1.4 System application schematic diagram	3

2. Module Hardware Specifications	4

2.1 Module Hardware Pin Definitions	4

2.2 Module hardware packaging dimensions	5

3. Communication Protocol Description	6

3.1 Distributed Ranging Frame	6

3.2 Node configuration frame	7

3.3 OTA upgrade frame	7

- System Introduction

  - System Introduction

- Makerfabs Electronics' DSTO series distributed ranging system is a high-performance,secure ranging system independently developed based on UWB (Ultra-Wideband)technology. This system boasts excellent ranging accuracy down to 10 centimetersand provides real-time distance information at a refresh rate of up to 500Hz, achieving millisecond-level ultra-fast response. lts distributed architecture theoretically allows for unlimited system node capacity and enables data packet output via standard serial ports, significantly reducing intergration complexity.

| Distributed ranging: UWB-DSTO moudle |  |
| --- | --- |
| Product Images |  |
| Coverage | Open view with visibility up to 200 meters |
| External dimensions | 28mm*19mm*3mm |
| Power supply method | DC3.3v |
| Antenna design | Built-in antenna |
| Operating temperature | -20℃~60℃ |
| Operating frequency | CH9 band 8987.2MHz |
| Refresh rate | 500Hz |
| System capacity | System capacity is unlimited / Capacity in the same area is 20(Contact for capacity adjustments) |

  - System Features

- Flexible network, unlimited capacity: The distributed architecture supports unlimited node expansion, allowing system capacity to grow on demand and easily handle future business upgrades.

- Ultra-fast response, imperceptible latency: A 500Hz refresh rate ensures instantaneous data feedback within 2 milliseconds, meeting the most demanding real-time control and safety application requirements.

- Intelligent Networking, Plug and Play: Nodes possess autonomous discovery and networking capabilities, enabling rapid deployment and system self-healing, greatly simplifying operation and maintenance.

- Exceptional Accuracy, Reliable Foundation: High-precision ranging with 10-centimeter accuracy and stable performance, providing reliable distance information even in multipath interference environments.

- Ultra-Long Ranging, Unrestricted Operation: A maximum open-space visual range of 200 meters, perfectly suited for large indoor and outdoor scenarios, providing ample operating space.

  - Application objects

- Drone formation applications

- Forklift anti-collision applications

- AGV/AMR cluster scheduling and anti-collision applications

- Robot collision-free collaborative applications

- Mining truck anti-collision and collaborative applications

  - System application schematic diagram

Schematic diagram of distributed distance measurement.

- Module Hardware Specifications

  - Module Hardware Pin Definitions

- Module Pinout Description 1

- Module Pinout Description 2

| Pin | Function |
| --- | --- |
| 1/2/10/11/19/20/24/34/35 | GND |
| 3 | UART_Rx Pin speed 115200 |
| 4 | Download CLK |
| 5 | Download DIO |
| 6/12/13/14/15/16/21/23/ 25/26/27/28/29/30/31/32 | NC must be suspended |
| 7 | UART_Tx Pin speed 115200 |
| 8、9、22 | 3V3 power supply |
| 17 | Output pins(status indicator lights) Configured: Fast flash(10hz) Not configured: Show flash(1hz) |
| 18 | - |
| 33 | Output pin (Networking indicator LED) Networked: Slow flashing (1Hz) Not networked: Off (Low level) |

  - Module hardware packaging dimensions

Module size specifications

- Communication Protocol Description

  - Distributed Ranging Frame

| Field name | Byte type | Length (Byte) | Index subscript | Function Description |
| --- | --- | --- | --- | --- |
| head | uint8_t | 1 | 0 | Fixed header 0x2A |
| cmd_len | uint16_t | 2 | 1 | Data frame length (length of the fields: cmd_saddr saddr-reserve) |
| cmd_saddr | uint64_t | 8 | 3 | Self-assigned long address (8 bytes reserved) |
| cmd_daddr | uint64_t | 8 | 11 | Recipient's full address (8 bytes reserved) |
| cmd_type | uint8_t | 1 | 19 | Command frame types (0x02: Node configuration frame, 0x10: OTA upgrade frame, 0x71: Distributed ranging frame) |
| cmd_direct | uint8_t | 1 | 20 | Command frame direction (1: Engine request, 2: Device response, 3: Device request, 4: Engine response, 5: Engine report, 6: Device report) |
| dev_time | uint32_t | 4 | 21 | Device communication time |
| dev_devID | uint32_t | 4 | 25 | Device unique ID |
| dev_role | uint16_t | 2 | 29 | Device roles |
| dev_capacity | uint8_t | 1 | 31 | Device capacity |
| dev_sn | uint32_t | 4 | 32 | Device reporting serial number (sn) |
| dev_onlineCnt | uint8_t | 1 | 36 | Number of devices currently online |
| other(1..20) para | uint64_t | 8 | 37, 47, ... | Basic information about other devices; this is a bit structure. |
| other(1..20) dist | uint16_t | 2 | 45, 55, ... | The distance parameters for other devices are in centimeters (cm). |
| check | uint8_t | 1 | * | Checksum (Field: Checksum before cmd_saddr-check)[Note 2: Checksum calculation program] |
| foot | uint8_t | 1 | * | Fixed trailer byte: 0x23 |

  - Node configuration frame

| Field name | Byte type | Length (Byte) | Index subscript | Function Description |
| --- | --- | --- | --- | --- |
| head | uint8_t | 1 | 0 | Fixed header 0x2A |
| cmd_len | uint16_t | 2 | 1 | Data frame length (length of the fields: cmd_saddr saddr-reserve) |
| cmd_saddr | uint64_t | 8 | 3 | Self-assigned long address (8 bytes reserved) |
| cmd_daddr | uint64_t | 8 | 11 | Recipient's full address (8 bytes reserved) |
| cmd_type | uint8_t | 1 | 19 | Command frame types (0x02: Node configuration frame, 0x10: OTA upgrade frame, 0x71: Distributed ranging frame) |
| cmd_direct | uint8_t | 1 | 20 | Command frame direction (1: Engine request, 2: Device response, 3: Device request, 4: Engine response, 5: Engine report, 6: Device report) |
| config_cmd | uint8_t* | * | * | Configuration command set [Note 3: Base station configuration command set] |
| check | uint8_t | 1 | * | Checksum (Field: checksum of cmd_ saddr-config_cmd) [Note 2: Checksum calculation program] |
| foot | uint8_t | 1 | * | Fixed trailer byte: 0x23 |

  - OTA upgrade frame

| Field name | Byte type | Length (Byte) | Index subscript | Function Description |
| --- | --- | --- | --- | --- |
| head | uint8_t | 1 | 0 | Fixed header 0x2A |
| cmd_len | uint16_t | 2 | 1 | Data frame length (length of the fields: cmd and saddr-reserve) |
| cmd_saddr | uint64_t | 8 | 3 | Self-assigned long address (8 bytes reserved) |
| cmd_daddr | uint64_t | 8 | 11 | Recipient's full address (8 bytes reserved) |
| cmd_type | uint8_t | 1 | 19 | Command frame types (0x02: Node configuration frame, 0x10: OTA upgrade frame, 0x71: Distributed ranging frame) |
| cmd_direct | uint8_t | 1 | 20 | Command frame direction (1: Engine request, 2: Device response, 3: Device request, 4: Engine response, 5: Engine report, 6: Device report) |
| OTA_cmd | uint8_t* | * | * | OTA commands (developers can ignore this) |
| check | uint8_t | 1 | * | Checksum (Field: checksum of cmd_ saddr-OTA_cmd) [Note 2: Checksum calculation program] |
| foot | uint8_t | 1 | * | Fixed trailer byte: 0x23 |

[Note 1: Bit structure]

typedef struct {

uint64 t dev_mode:2;	// Device Mode

uint64_t dev_id:32;		// devID

uint64_t dev_type:3;	// Device Type

uint64 t done:1;		// ignore

uint64 t inNet:1,		// Is networking enabled?

uint64 t reserve:25;}devPara t;	// Reserved

} devPara_t;

[Note 2: Verification calculation program]

Static uint8_t get_Xor_CRC(uint8_t *bytes, int offset, int len)

{

uint8_t xor_crc = 0;

Int i;

for (i = 0l i < len; i++) {

xor_crc ^= bytes[offset + i];

}

return xor_crc;

}

[Note 3: Node Configuration Command Set]

| Command | Meaning | Examples |
| --- | --- | --- |
| restart | Reset | Reset |
| restore | Restore to factory settings | Restore to factory settings |
| save | Save | Save settings |
| getver | Get the version |  |
| setcfg (x1) (x2) (x3) (x4) (x5) (x6) | Set up configuration | setcfg 0 0 50 1122 0 1f x1: 0-10000 x2: Device role (-1: not set, 0: tag, 1: base station. Default: -1 not set) x3: Device capacity (20 devices in the same area) x4: Device Network x5: Device PA function (0: Off, 1: On. Default: 0 Off) x6: Device power setting (0-120) |
| getcfg | Get configuration |  |
| setAntD(x1) | Set antenna delay | setAntD 3454 x1: Antenna delay parameter (default: 3454) |
| getAntD | Get antenna delay |  |