Software Usage

- Open and connect to the host computer

Note: When data is transmitted, garbled characters may appear in the [Serial Port/Network Assistant]. This data is not actually garbled, but rather hexadecimal data.

- Distance display interface

- Console configuration interface

[Table Parameters]

Device ID: devID of other devices (user-defined)

Device Distance: Distance from other devices to this device

Device Capacity: Type of other devices

Device Network: Operating mode of other devices

Device Power: Voltage of other devices

Device PA Enabled: Charging status of other devices

- Device configuration interface

Parameter Description:

【Device Parameters】

Device ID: Device ID, no upper limit 0-100000 (upper computer range, actually an int type)

Device Role: Default

Device Capacity: Default setting is 20, if you have specific capacity requirements, please contact our company for modification.

Device Network: Devices with the same network ID can measure distances to each other; different networks are not allowed to measure distances.

Device Power: Device power setting (0-120)

Device PA Enable: Power amplifier setting (0: Off 1: On)

【Calibration Parameters】

Device Antenna Delay: Adjusts the error in distance measurement during module ranging (default is sufficient)

【Version Parameters】

Device Production Code: The unique MAC address of the device at the time of manufacture

Device Production Time: The manufacturing date of the device

Software/Hardware Version: Base station software version number

Reading Parameters Instructions:

Step 1: Click [Read All Parameters]

Writing Parameters Instructions:

Step 1: Click [Read Parameters]

Step 2: Modify the parameters

Step 3: Click [Import Parameters]

Step 4: Click [Save Parameters]